Systems Architecture

Autonomous Localized Mobile Photography Workflow

Lenovo Yoga 910-13IKB Glass (Star Wars Special Edition)

Revision: 4.0  |  April 10, 2026

Development Platform: Claude Code (Multi-Agent)

# 1. Executive Summary

This document is the revised technical specification for an autonomous ingest-to-edit photography system targeting the Lenovo Yoga 910-13IKB Glass (Star Wars Special Edition). Development uses Claude Code with a multi-agent architecture (@architect, @engineer, @devops).

Revision 4.0 corrects the target hardware profile from the Yoga 920 (i7-8550U, 16 GB, Thunderbolt 3) to the actual Yoga 910 (i7-7500U, 8 GB, USB 3.1 Gen 1). This change impacts memory budgets, core count assumptions, inference benchmarks, USB topology for SSD cartridges, and eliminates 4K display scaling requirements.

# 2. Hardware Delta: Yoga 920 vs. Yoga 910

The following table documents every specification change and its downstream impact on the architecture.

| Spec Item | Previous (Yoga 920) | Actual (Yoga 910-13IKB Glass) | Impact |
| --- | --- | --- | --- |
| CPU | i7-8550U (4C/8T, 1.8-4.0 GHz, Kaby Lake R) | i7-7500U (2C/4T, 2.7-3.5 GHz, Kaby Lake) | Half the cores. KPM-1.2 inference target may need re-baselining. AVX2 still supported -- ONNX path unchanged. |
| RAM | 16 GB DDR4 | 8 GB DDR4-2133 | NFR-2.2 container limit must be tightened. 2 GB container + OS + Docker on 8 GB total is tight. Revised to 1.5 GB container max. |
| USB-C (Left) | 2x Thunderbolt 3 | 1x USB 3.1 Gen 1 + 1x USB 2.0 (charging only) | Single data-capable USB-C port for SSD cartridges. No Thunderbolt. udev rules simplified to one SSD port. |
| USB-A (Right) | 1x USB 3.0 | 1x USB 3.0 (Always On) | Unchanged. SD card reader ingest path remains the same. |
| Display | 4K (3840x2160) | FHD (1920x1080) | No display scaling needed. Selkies 200% scaling section removed. Darktable UI runs at native resolution. |
| Wireless | Not specified | Lenovo 2x2 AC + Bluetooth 4.1 | System is offline (NFR-2.1) but Wi-Fi driver stability still matters for OS updates and initial setup. |
| Storage | Not specified | 256 GB SSD | Limited internal storage reinforces the SSD cartridge architecture (FR-1.9). OS + Docker images must fit in ~200 GB. |

# 3. Target Host Hardware

Platform: Lenovo Yoga 910-13IKB Glass (Star Wars Special Edition)

Model: 80VG series

| Component | Specification | Constraint / Notes |
| --- | --- | --- |
| CPU | Intel Core i7-7500U (2C/4T, 2.7 GHz base, 3.5 GHz turbo) | AVX2 supported. Dual-core limits parallel processing. All benchmarks target this CPU. |
| RAM | 8 GB DDR4-2133 (soldered) | Not upgradeable. Container RSS limit revised to 1.5 GB (KPM-1.3). Host OS needs ~2-3 GB. |
| USB-C Left (rear) | 1x USB 2.0 (charging + data) | Charging port. Not suitable for high-speed SSD -- data rate limited to USB 2.0 (480 Mbps). |
| USB-C Left (front) | 1x USB 3.1 Gen 1 (5 Gbps) | Primary SSD cartridge port. Single port means one cartridge at a time. |
| USB-A Right | 1x USB 3.0 Always On (5 Gbps) | SD card reader ingest path. Always-On means powered even when laptop is off. |
| Display | 13.9" FHD IPS (1920x1080) | No scaling required. Darktable and zenity dialogs render at native resolution. |
| Storage | 256 GB M.2 SSD | OS + Docker + tools only. All photo libraries live on external SSD cartridges. |
| Wireless | Lenovo 2x2 802.11ac + BT 4.1 | Offline system (NFR-2.1). Wi-Fi used only for initial OS setup and updates. |
| Camera | 720p HD webcam | Not used by this system. |

### 3.1 Linux-Specific Considerations

- Wi-Fi: Lenovo 2x2 AC adapter may require firmware updates or module configuration on Ubuntu 24.04 LTS.

- Tablet Mode: Yoga 910 uses a watchband hinge for 360-degree rotation. Tablet mode support requires appropriate kernel modules for screen auto-rotation.

- Display: FHD at 13.9" -- no scaling needed. Default DPI settings should work out of the box.

- USB-C: Only the front left USB-C port is USB 3.1. udev rules must distinguish it from the rear USB 2.0 charging port.

# 4. Revised Requirements

Functional requirements are unchanged from v3. Non-functional requirements and KPMs are updated to reflect the Yoga 910 hardware.

## 4.1 Non-Functional Requirements (Revised)

| ID | Description | Specification | Change from v3 |
| --- | --- | --- | --- |
| NFR-2.1 | Internet Independence | 100% offline. All model weights, pip packages, and libraries pre-installed. | No change. |
| NFR-2.2 | Resource Efficiency | Total container RSS <= 1.5 GB. CPU affinity capped at 80%. Host OS reserved: 2.5 GB minimum. | Tightened from 2 GB container on 16 GB system to 1.5 GB on 8 GB system. Added host reservation. |
| NFR-2.3 | Database Portability | Darktable library.db + user config reside on external SSD, not host filesystem. | No change. |
| NFR-2.4 | Interactive UI Prompts | zenity dialogs notify user if SD card inserted with no SSD connected. | No change. |

## 4.2 Key Performance Measures (Revised)

| KPM | Metric | Target | Owner | Change from v3 |
| --- | --- | --- | --- | --- |
| KPM-1.1 | Ingest Latency | >= 80% USB 3.0 bandwidth | @devops | No change. USB 3.0 port is the same. |
| KPM-1.2 | Inference Speed | <= 2.5s per image (Florence-2 INT8) | @engineer | Relaxed from 1.5s. Dual-core i7-7500U has ~40% less multi-thread throughput than the quad-core i7-8550U. |
| KPM-1.3 | Memory Stability | RSS <= 1.5 GB for analyzer container | @engineer | Tightened from implicit 2 GB. 8 GB total RAM leaves no margin. |
| KPM-1.4 | Data Integrity | Zero SQLite corruption over 50 eject cycles | @devops | No change. |

NOTE: KPM-1.2 is provisionally set at 2.5s pending benchmarking on the actual i7-7500U. If the INT8 quantized Florence-2 model achieves faster inference, the target should be tightened back toward 1.5s.

# 5. Memory Budget (8 GB System)

With only 8 GB of total RAM, memory allocation must be explicit. The following budget ensures the system remains stable under load.

| Component | Allocation | Notes |
| --- | --- | --- |
| Linux kernel + OS services | ~1.0 GB | Minimal Ubuntu 24.04 server or Debian Stable footprint. |
| Docker daemon | ~0.3 GB | Overhead for container runtime. |
| Darktable container (Selkies UI) | ~2.0 GB | Darktable with a moderate library open. Largest single consumer. |
| Analyzer container | 1.5 GB (hard limit) | KPM-1.3. Florence-2 INT8 model + image buffers + analysis pipeline. |
| System buffer/cache | ~3.2 GB | Remaining RAM used by kernel for filesystem caching. Reclaimable under pressure. |
| TOTAL | 8.0 GB | No swap recommended -- SSD wear concern on 256 GB internal drive. |

# 6. USB Port Topology and udev Strategy

The Yoga 910's limited USB port layout requires careful assignment. Unlike the Yoga 920 with dual Thunderbolt, only one USB-C port is viable for high-speed SSD data transfer.

| Port | Location | Speed | Assignment | udev Strategy |
| --- | --- | --- | --- | --- |
| USB-C (rear) | Left side, rear | USB 2.0 (480 Mbps) | Power/charging only | Exclude from SSD matching rules. Do not mount storage from this port. |
| USB-C (front) | Left side, front | USB 3.1 Gen 1 (5 Gbps) | SSD cartridge | Match by %s{serial}. Map to /dev/photo-ssd-ID. Single cartridge at a time. |
| USB-A | Right side | USB 3.0 (5 Gbps) | SD card reader | Match by device class. Trigger rsync ingest on insertion. |

CONSTRAINT: With only one data-capable USB-C port, simultaneous SSD cartridge swap and ingest from SD is supported (SSD on USB-C, SD reader on USB-A), but two SSD cartridges cannot be connected simultaneously.

# 7. Claude Code Project Structure

photo-workflow/

CLAUDE.md                  # Project spec + task sequencing

pyproject.toml             # Package definition, dependencies, entry points

.claude/agents/            # Multi-agent definitions

architect.md             # Read-only, opus model

engineer.md              # All tools, sonnet model

devops.md                # All tools, sonnet model

src/

photo_workflow/

__init__.py

pipeline.py            # AnalysisPipeline orchestrator

ingest.py              # FR-1.1: rsync trigger logic

grouping.py            # FR-1.2: spatio-temporal clustering

dedup.py               # FR-1.3: dHash dedup (Hamming <= 2)

sharpness.py           # FR-1.4: Laplacian variance scoring

composition.py         # FR-1.5: rule-of-thirds + saliency

exposure.py            # FR-1.6: 11-zone luminance entropy

naming.py              # FR-1.7: Florence-2 INT8 ONNX naming

darktable_bridge.py    # FR-1.8: SQLite + XMP sidecar sync

cartridge.py           # FR-1.9: SSD volume management

scripts/

safe_eject.sh            # FR-1.10: WAL flush + unmount

manage_ssd.sh            # SSD detection + Docker volume remap

install_udev.sh          # udev rule installer

deploy/

Dockerfile               # Container (Debian Stable / Ubuntu 24.04)

docker-compose.yml       # Service orchestration + volume mapping

udev/

99-photo-ssd.rules     # SSD serial matching (USB-C front only)

99-photo-sd.rules      # SD card auto-ingest trigger

tests/

fixtures/                # Test images, mock library.db

test_pipeline.py

test_dedup.py

test_sharpness.py

test_darktable.py

test_memory.py           # RSS monitoring for KPM-1.3

models/

florence2_int8/           # Pre-quantized ONNX weights (vendored)

# 8. Agent Roster

| Agent | Model | Tools | Scope | Memory |
| --- | --- | --- | --- | --- |
| @architect | opus | Read, Grep, Glob (read-only) | CLAUDE.md, architecture, interface contracts, dependency decisions | user scope |
| @engineer | sonnet | All tools | src/, tests/, models/ | project scope |
| @devops | sonnet | All tools | deploy/, scripts/ | project scope |

# 9. Execution Roadmap

| Stage | Owner | Scope | Acceptance Criteria |
| --- | --- | --- | --- |
| 1. Scaffold | @architect | Project init, CLAUDE.md, pyproject.toml, directory structure, agent definitions | pip install -e . succeeds; pytest discovers all test files; claude agents lists all 3 |
| 2. Core Engine | @engineer | FR-1.2 through FR-1.6: grouping, dedup, sharpness, composition, exposure | All tests pass; memory profiler < 500 MB per module |
| 3. Inference + Bridge | @engineer | FR-1.7 (Florence-2 naming) + FR-1.8 (Darktable SQLite/XMP) | KPM-1.2 <= 2.5s/image on i7-7500U; zero DB corruption |
| 4. Host Integration | @devops | FR-1.1, FR-1.9, FR-1.10: udev, SSD cartridge, safe eject, Docker | KPM-1.1 >= 80% USB 3.0 BW; KPM-1.4 50 safe removals |
| 5. Parallel Build | @engineer + @devops | Agent team: concurrent Stage 2-4 work (requires CLAUDE_CODE_EXPERIMENTAL_AGENT_TEAMS=1) | All individual stage criteria met |
| 6. Integration | @architect (lead) | Full pipeline integration, end-to-end testing on Yoga 910 | All KPMs verified on hardware; pipeline runs SD-to-Darktable autonomously |

# 10. Intelligence Engine Specification

The inference subsystem targets the i7-7500U's AVX2 instruction set. Dual-core constraint means inference is largely single-threaded unless onnxruntime's intra-op parallelism is configured for 2 physical cores.

- Runtime: onnxruntime (CPU provider, AVX2 optimized). No CUDA, no OpenVINO.

- Model: Florence-2-base-ft, pre-exported ONNX from onnx-community/Florence-2-base-ft, quantized to INT8.

- Quantization: Dynamic INT8 via onnxruntime.quantization. Weights in models/florence2_int8/.

- Memory budget: Model + runtime + active image buffers must fit within 1.5 GB RSS (KPM-1.3).

- Threading: Set ORT_NUM_THREADS=2 to match physical core count. Avoid oversubscription on dual-core.

# 11. Development Workflow Notes

### 11.1 What Claude Code Handles vs. Manual Steps

| Claude Code (Automated) | Manual (Requires Yoga 910 + Linux) |
| --- | --- |
| Python module implementation + tests | udev rule testing on Yoga 910 USB topology |
| Shell script generation (safe_eject, manage_ssd) | ONNX weight acquisition and INT8 quantization |
| Dockerfile + docker-compose authoring | USB-C port differentiation (front vs. rear) |
| SQLite schema fixture generation | Physical SSD mount/eject cycle testing |
| Linting, formatting, type checking | Wi-Fi driver configuration |
| pytest execution and iteration | Darktable UI verification at native FHD resolution |

### 11.2 Removed from v3

- 4K display scaling: Not applicable. Yoga 910 has FHD display. Selkies 200% scaling configuration removed.

- Dual Thunderbolt SSD: Not applicable. Single USB 3.1 Gen 1 port. No simultaneous dual-cartridge support.

- pinctrl_tigerlake: That kernel module is for Tiger Lake (11th gen). Yoga 910 uses Kaby Lake. Tablet mode kernel modules TBD during Linux install.

# 12. References

1. pHash in Python | Hashing and Validation -- SSOJet (accessed April 7, 2026)

2. Open source image recognition with Luminoth | Opensource.com (accessed April 7, 2026)

3. Best lightweight Linux distro of 2025 -- TechRadar (accessed April 7, 2026)

4. How to compact the library? -- darktable -- discuss.pixls.us (accessed April 7, 2026)

5. Meet BLIP: The Vision-Language Model Powering Image Captioning -- PyImageSearch (accessed April 7, 2026)

6. Intel Core i7-7500U Specifications -- Intel ARK (accessed April 10, 2026)

7. Lenovo Yoga 910-13IKB Convertible Review -- NotebookCheck (accessed April 10, 2026)

8. onnx-community/Florence-2-base-ft -- Hugging Face (accessed April 10, 2026)